CARPET LIGHT — SHOW ORDER & CONFIG MENU REFERENCE

LIGHT SHOW ORDER

S advances 1->2->3->4->5->1. Enc right/left advances/retreats the show's own variation, wrapping. Enc right/left below is per the code's own convention -- NOT yet verified against real wiring (flip if backwards).

Global (any show, any variation)

S next show -- M no effect -- L enter config -- XL lights on/off

D UV on/off (Equalizer: triple-strobe toggle instead)

Enc right next variation, Enc left previous

1 Nightrider (3 variations)

ManualHue -- Pot sets hue directly

AutoHueCycle -- Pot sets flood auto-hue rate, 0.25-2Hz

AutoWithSound -- Pot sets flood auto-hue rate; china flashes on bass hits

2 Flame (2 variations)

Waterflames -- Pot sets fire-sim speed

Flames -- Pot sets fire-sim speed

3 Equalizer (3 variations)

VU meter -- Pot sets live peak threshold; D toggles triple-strobe

new_standard -- Pot sets cycle length, 2-128 bass beats (log); D toggles triple-strobe

pixel_war -- Pot sets cycle length, 2-128 bass beats (log); D toggles triple-strobe

4 SpeedStripes (2 variations)

default -- Pot has no effect

zebra -- Pot sets black-stripe width, 0-40ft

5 Lighthouse (2 variations)

default -- Pot sets rotation-speed ceiling; china bass-hit white strobe

No Strobe -- Pot sets rotation-speed ceiling; no china strobe

CONFIG MENU STRUCTURE

L enters config (always @ ① Brightness). M advances screen, wraps ①->②->③->①. S advances subsetting. D saves+exits. L cancels, no save. Enc right/left below is per the code's own convention -- NOT yet verified against real wiring (flip if backwards).

① Brightness (3 sub) -- Pot adjusts each live value

Global -- 0-100%

Headlight -- 50-100%

China -- 0-100%

② Audio (6 sub)

NoiseFloor/PeakThreshold -- S swaps which Pot adjusts

HitDecay -- Pot sets 0-1000ms

Foresight -- Pot 0-1000ms; Enc R/L next/prev hit-predict style (off/exp/machine-gun/drum, wraps)

AGC -- Enc R advances Off->Band->Full, Enc L back toward Off (clamped); Pot n/a

SndReact -- Enc R = on, Enc L = off (direct pick); Pot n/a

AutoPk -- Enc R advances Off->Full->Bin, Enc L back toward Off (clamped); Pot n/a

③ PowerTest (3 sub) -- Enc adjusts each; Pot n/a

Hue

Saturation

Brightness

Generated 2026-08-27 06:44 PM PDT (Pacific) by tools/gen_lightshow_config_reference.py